REQUIREMENTS SPECIFICATION

VERSION 1.0 18 March 2025

European openBIM Forum

openBIM Harmony

PART 2.1 EXCHANGE INFORMATION REQUIREMENTS

BUILDINGS

Contents

- Exchange Information Requirements

  - General

  - Required use of information

- Level of Information Need

  - General

  - Prerequisites

  - Requirements for geometrical information

  - Requirements for alphanumerical information

  - Explanation of requirements for alphanumerical information

- Method for project information model (PIM)

- The project's information standard

  - General

  - Spatial breakdown structure

  - Naming conventions

- The project’s information production method and procedures

  - General

  - Routines

- Project’s information protocol

- Generic project stages

Explanation of color and type

Black text = Part of framework. Can be distributed as is.

Red text = Example. Shall be changed to project information before distribution

Red italic text = Guidance to fill out information. Shall be deleted before distribution

1.	Exchange Information Requirements

1.1	General

Exchange Information Requirements are a compilation of information requirements for an agreed information delivery. Exchange Information Requirements are abbreviated to EIR.

The Exchange Information Requirements is an aggregation and detailed specification of overall requirements from three sources of requirements:

- The organizational information requirements (OIR, Organization Information Requirements)

- The Asset Information Requirements (AIR, Asset Information Requirements).

- The Project Information Requirements (PIR, Project Information Requirements).

The EIR specifies information to be exchanged between appointed parties, the Appointing Party and other agreed stakeholders, and ensures that all parties have the necessary data for their tasks. The Appointing Party is responsible for establishing and maintaining the EIR. Source: EN ISO 19650-1

1.2	Required use of information

1.2.1	General

Information requirements specify what information is needed, when it is required, in what format, and who the recipient is. These requirements may originate from the appointing party's organizational level (OIR), asset manager (AIR), or the project (PIR). High-level requirements are translated into specific information deliverables, referred to as EIR requirements. It is recommended to include the original requirements, from which the EIR is derived, in the tender documents shared with potential lead appointed parties. This provides appointed parties with a better understanding of the purpose of the information and enables them to contribute optimally to meeting the needs.

The required use of BIM is defined in clause 1.2.2 to 1.2.8.

1.2.2	EIR-01 Project overview

Report on project status about the progression, financial frame, deviations and risks. This includes making the models available to the project’s participants.

1.2.3	EIR-02 Modelling and model-based interaction

BIM shall be used as the primary method for documentation and decision-making. Design disciplines develop and maintain models that represent the project's development.

Models are developed to support the project's decision-making. Models should represent the maturity of the project according to the life-cycle standard adopted.

BIM is used as the primary method for documentation and decision-making. Design disciplines develop and maintain models that represent the project's development.

1.2.4	EIR-03 Interdisciplinary coordinated and controlled information

Appointed parties compile information models, and geometrical and functional deviations are registered. Information models are checked according to geometrical and alphanumerical requirements. Deviations are reported and corrected. Control is documented.

1.2.5	EIR-04 Cost estimation information

Model-based cost estimation as a means to cost-driven design. The project will use models for frequent quantity take-off for cost estimation.

1.2.6	EIR-05 Production information

Appointed parties shall support optimizing the use of models to procure, plan and execute production. This includes coordinating with contractors and subcontractors on information deliverables.

1.2.7	EIR-06 Completion and handover

Appointed parties shall complete and handover as-built information and documentation as part of the facility management information. It also includes PIM set-up and information control documentation.

2	Level of Information Need

2.1	General

This specifies the information to be delivered.

2.2	Prerequisites

All modeling disciplines are required to contribute to the agreed-upon information and support model-based project execution. This applies to the five primary design disciplines: architectural, landscape architectural, structural, plumbing, mechanical, and electrical.

If the responsibility for design is transferred to other parties, such as subcontractors, these parties are also required to deliver information that complies with these requirements.

All information deliveries must undergo quality assurance and be submitted by the agreed milestones. These milestones are strategically set to support decision-making processes and ensure smooth progress into production. By adhering to these milestones, the project maintains a structured workflow, enabling timely evaluations and adjustments as needed.

Information is exchanged at predetermined intervals to facilitate interdisciplinary coordination and control. This structured exchange allows for gradual progress toward full interdisciplinary alignment, minimizing conflicts and enhancing overall project efficiency. Regular coordination ensures that all disciplines are synchronized, leading to a cohesive and integrated project outcome.

2.3	Requirements for geometrical information

Description of geometrical information that can be expressed using the independent aspects: Detail, dimensionality, location, appearance and parametrical behavior.

The geometry shall be representative of the physical product. It shall be possible to see what the object represents. The outer boundaries of the object shall be representative for the purpose of spatial coordination. The geometry of the individual objects shall not be too detailed as it can slow down the aggregated model.

The model use an absolute location reference, either global coordinates or a local point of origin.

2.4	Requirements for alphanumerical information

Description of detail and extent of information that can be expressed using characters, digits and symbols or tokens such as mathematical symbols and punctuation marks. Alphanumerical information can be used to either specify identification or properties of an object.

Table 1 - Information level - Requirements for alphanumerical information

You find this table in the spreadsheet version “Harmony_2-2_EIR-BEP_V1”, “tab EIR_LOIN”

| Requirement1 | ARC2 | LAN2 | STR2 | PLU2 | MEC2 | ELE2 | PS 43 | PS 5 | PS 6 | PS 7 | PS 8 | PS 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Object type name | X | X | X | X | X | X | X | X | X | X | X | X |
| Object type descriptive name | X | X | X |  |  |  |  |  | X | X | X | X |
| Object occurrence name | X | X | X | X | X | X |  |  |  |  | X | X |
| Location system |  |  |  | X | X | X |  |  | X | X | X | X |
| System code | X | X | X | X | X | X |  | X | X | X | X | X |
| Process Status Code (MMI) | X | X | X | X | X | X | X | X | X | X | X | X |
| Duplicate object | X | X | X | X | X | X |  | X | X | X | X | X |
| External | X | X | X |  |  |  |  |  | X | X | X | X |
| Fire rating | X | X | X |  |  |  |  |  | X | X | X | X |
| Acoustical property | X | X |  |  |  |  |  |  | X | X | X | X |
| U-value | X | X |  |  |  |  |  |  | X | X | X | X |
| Load-bearing |  |  | X |  |  |  |  |  | X | X | X | X |
| 1 Requirements are explained in the table below, 2.1.1 Explanation of requirements for alphanumerical information. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 ARC = Architectural, LAN = Landscape architectural, STR = Structural engineering, PLU = Plumbing Engineering, MEC = Mechanical engineering, ELE = Electrical engineering |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 PS = Project Stage. The project stages refer to ISO 29481-1 as a common reference. Read more in clause 3.3 Project stages. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Applies only to elements with Pset_XxxCommon. XxxCommon indicates that requirements for the property are found on property sets for specific object classes, e.g. IfcWallCommon. Xxx is used to indicate that it applies to several object classes, e.g. columns, beams, walls, decks, doors, windows etc. |  |  |  |  |  |  |  |  |  |  |  |  |

2.5	Explanation of requirements for alphanumerical information

This table explains the requirements for alphanumerical information specified in table 1. The information shall be exchanged in a standardized and consistent manner and shall be quality assured.

2.5.1	Object type name

Identifies the type of the element. Identification of type is important for cost analysis, building specification, linking to product documentation etc.

Name is specified as an attribute of IfcRoot. 
Elements are the common term for all object classes that represent physical building objects. e.g. pillar, staircase, pump and lamp.

Ideally the name is allocated on the entity’s object type. However, some authoring tools use Name on the object type for other purposes. In that case it may be necessary to allocate the object type name on the object occurrence.

The object type name is specified using IfcRoot.Name on the object's type.

2.5.2	Object type descriptive name

Descriptive names are used e.g. by calculator and description manager to understand what the object is. 
For technical disciplines: If the object type code is enough to indicate the object, this is not needed.

The object type name is specified using IfcRoot.Description on the object's type.

2.5.3	Object occurrence name

Identification code for the element’s occurrence. This information is necessary for document information specifically for the separate occurrence of the object. This is typically a component class code followed by a serialized number.

The object type name is specified using IfcRoot.Name on the object's instance level.

2.5.5	Location of (technical) system

This is a project specific code communicating the location of the primary system. This usually applies to technical systems, but is can also apply to grouping building components into a functional system e.g. “building envelope“. The code is not supported by IFC. It requires a userdefined property set and property. E.g. EUHARM_Reference_RefPriSysLoc

2.5.6	System code

The code of the primary system that the object belongs to. This is important to communicate the objects relation to a system. This usually applies to technical systems, but is can also apply to grouping building components into a functional system e.g. “building envelope“. The code is not supported by IFC. It requires a userdefined property set and property. E.g. EUHARM_Reference.RefPriSysOcc

2.5.7	Process Status Code (MMI)

A code communicating the model’s maturity, on object level, according to the project’s decisions and QA processes. The code is not supported by IFC. It requires a userdefined property. E.g. EUHARM_Process.ProcessStatus.

2.5.8	Duplicate object

The code communicates that another discipline is responsible for the object. The object is included in the model to show appearance, location or the like, but technical information about the object is described in the model to the discipline responsible. The value for the property is filled in with the discipline abbreviation of the discipline responsible for the object, for example ARC, STR, PLU, MEC, ELE, LAN, etc. The code is not supported by IFC. It requires a userdefined property. E.g. EUHARM_Process.DuplicateOwnedBy

2.5.9	External

Specifies if an element is part of the building envelope/outside a construction or if it is internal.

The property is specified using the Common property set available for the object class Pset_XxxCommon.IsExternal (the Xxx is a placeholder for the object class e.g. Pset_WallCommon or Pset_DoorCommon).

2.5.10	Fire rating

Fire rating refers to how well a material, component or building part resists fire or limits the spread of flames and smoke. Applies typically to enclosing element classes such as wall, slabs, windows, doors etc. Fire rating is indicated in accordance with the national fire safety classification. The property is specified using the Common property set available for the object class Pset_XxxCommon.FireRating

2.5.11	Acoustical property

The property indicates the sound transmission resistance of this object using an index value (rather than providing full sound absorption values). It is specified in accordance with national building regulations. The property is specified using the Common property set available for the object class Pset_XxxCommon.AcousticRating

2.5.12	U-value

Heat transfer coefficient, often called U-value, is a measure of how well a building part, such as a wall, a window or a roof, insulates against heat loss. The U-value is measured in W/m²K (watts per square meter per degree Kelvin). Applies typically to element classes that are part of the building envelopes.

The property is specified using the Common property set available for the object class Pset_XxxCommon.ThermalTransmittance

2.5.13	Load-bearing

Indicates whether the object is intended to carry loads (TRUE) or not (FALSE). Applies to element classes such as beams, columns, walls etc. The property is specified using the Common property set available for the object class Pset_XxxCommon.LoadBearing

3	Method for project information model (PIM)

The following applies unless otherwise explicitly agreed and approved by the Appointing Party.

Table 2 – Method for project information model

You find this table in the spreadsheet version “Harmony_2-2_EIR-BEP”, tab “EIR_IPMP” (Project’s Information Production Methods and Procedures)

| No. | Requirement | Description |
| --- | --- | --- |
| 1 | IFC standard | Requirements apply to IFC4 and later versions. Models must be exported to the agreed version of IFC and object classes, attributes and properties must follow the IFC standard. The project must use the agreed version of the IFC standard for object classes, attributes and properties. Custom attributes and properties should only be established if the agreed version of the IFC standard does not meet needs. |
| 2 | Georeferencing | EPSG and no rotation of model relative to map. Common local project point of origin |
| 3 | Local point of origin | A common local point of origin must be used for all discipline models. The model should be in the first quadrant relative to the zero point to avoid negative numbers, which some modeling software can have problems with. The model should be placed at a distance from the zero point, so that all the model's parts are in the first quadrant. For object representing local point of origin se No. 10 in this table. |
| 4 | Gross area | A space object specifying the building story gross area(s). |
| 5 | SI units | SI units must be used. |
| 6 | Simplified model objects | Where model objects/discipline models are particularly detailed, beyond what is necessary for interdisciplinary planning and this weighs down the performance of assembled models, the appointing party can demand the delivery of a simplified model that is used for assembly purposes. This is perceived as part of the delivery offered. Simplified models for special analysis purposes, e.g. energy analysis, must be agreed at the conclusion of the agreement or is to be understood as an additional delivery. |
| 7 | Horizontal division of room areas | Rooms that extend over several floors must have a separate room object for each floor. |
| 8 | System affinity | Apply to technical disciplines. Objects belonging to the same system must be grouped in the same system. Either in IfcSystem or a relevant subcategory of IfcSystem. |
| 9 | Predefined subtypes | Use the IFC standard's subdivision of object classes where applicable to specify class and subclass. E.g. IfcCovering:FLOORING where Flooring is a predefined subtype of coverings specifying that the object represents a flooring. |
| 10 | Local point of origin and control objects | Local point of origin objects must be established, which lies in the local point of origin coordinate, so that it is possible to visually check that all models are correctly positioned. Two control points object must also be established to ensure that models are not rotated. Coordinates of local point of origin and control objects are documented in Part 3.1 BIM Execution Plan, clause Both zero point objects and control point objects must be represented by a "pie slice". The size of the “pie slice” should be such that it is clearly visible when looking at the entire model including the zero point and control objects on the screen. The bottom of the pie is at Z=0. Each discipline gets a “pie slice” of 30 degrees. Starting in the 1st quadrant: 01 ARK 0-30 degrees 02 LAN 30-60 degrees 03 STR 60-90 degrees 04 PLU 90-120 degrees 05 MEC 120-150 degrees 06 ELE 150-180 degrees 07 NN 180-210 degrees 08 NN 210-240 degrees 09 NN 240-270 degrees 10 NN 270-300 degrees 11 NN 300-330 degrees 12 NN 330-360 degrees NN = Discipline not specified. If more than 12 disciplines are needed, 12 additional pie slices can be established below these. The top of the pie slice is at height Z=0. Both zero point objects and control point objects must be represented by a "pie slice" from each designing discipline. The size of the pie slice should be such that it is clearly visible when looking at the entire model including the zero point and control objects on the screen. Local point of origin and control objects is recommended as IFC entities IfcAnnotationTypeEnum:SURVEY |

4	The project's information standard

4.1	General

Information shall be produced in line with the project’s information standard to ensure successful exchange with other technologies, using open data formats where possible.

4.2	Spatial Breakdown Structure

Information is delivered according to the model breakdown that follows EN ISO 16739 IFC. Storey (IfcBuildingStorey) will not normally be used in infrastructure projects. In construction, the model must relate to the project's common floor specification.

All disciplines shall use the same information for each attribute. Information about values is provided by the client.

Table 3 – Spatial Breakdown Structure

You find this table in the spreadsheet version “Harmony_2-2_EIR-BEP”, tab “EIR_IS” (Information Standard)

| Entity | Entity IFC | Identification | Identification attribute | Description |
| --- | --- | --- | --- | --- |
| Project | IfcProject | Project number | Name | Identified with appointing party's project number |
|  |  | Project name | LongName | Identified with appointing party's project name. |
| Site | IfcSite | Cadastral number | LandTitleNumber | Identified with cadastral number |
|  |  | Site numer | Name | Identified with appointing party's site number |
|  |  | Site name | LongName | Identified with appointing party's site name |
| Building | IfcBuilding | Building number | Name | Identified with appointing party's building number |
|  |  | Building name | LongName | Identified with appointing party's building name |
| Building storey | IfcBuildingStorey | Building storey number | Name | Identified with appointing party's building story number |
|  |  | Building storey name | LongName | Identified with appointing party's building story name |
| Space | IfcSpace | Space number | LongName | Identified with space number during building operation |
|  |  | Space name | Description | Identified with appointing party's/user organization's space name |
|  |  | Space function number | Name | Identified with appointing party's/user organization's space function number |

4.3	Naming conventions

4.3.1	Naming of model files and metadata

Model files must be named and specified with relevant metadata. See “Part 2.3 Common Data Environment”.

4.3.2	Naming of building storeys

The Lead Appointed Party must establish a system for naming floors in collaboration with the Appointing Party and other stakeholders, including the user representative and the authorities.

5	The project’s information production methods and procedures

5.1	General

The project's information production methods and procedures describe the details of how information is to be produced, processed and transferred throughout the project, to ensure effective communication and coordination between all parties involved.

5.3	Routines

5.3.1	Interdisciplinary coordination and model control

The Lead Appointed Party must establish and maintain routines for interdisciplinary quality assurance of models so that the quality of models complies with Exchange Information Requirements and that deviations are found and corrected.

Interdisciplinary quality assurance must be carried out on an ongoing basis as part of the exchange and coordination of models. Quality assurance must cover all models in the project, including design subcontractors.

Interdisciplinary quality assurance must also include final control in connection with the completion and handover of the project phase. This must include written documentation of the control carried out with a report from the model check and a complete checklist from the responsible disciplines.

Routines for interdisciplinary coordination and model control are made available for the project and are referred to from the BIM Execution Plan.

5.3.2	Process Status Coding

The appointed party must code objects to communicate maturity and suitability for use. The Lead Appointed Party must establish a system for this, make it available in the project and integrate it in interdisciplinary coordination and model control.

The Lead Appointed Party clarifies which codes are to be used and shares this with all relevant actors in the project.

5.3.3	Revision and version control

The Lead Appointed Party must establish a procedure for handling changes, revision, approval and distribution of information to ensure that information is in accordance with the project's requirements before distribution and that there is no doubt about the current revision. P procedure must include:

- Complementary and agreed description of steps in the procedure.

- Documentation of change history.

- Specification of approval roles and responsibilities.

- Methods for revision and distribution of updated files and models.

5.3.4	Training and skills development

The Lead Appointed Party must establish a plan for training project personnel what is to be delivered (EIR), how it is to be delivered (BEP). Everyone who must deliver on these requirements must be trained. This applies both at the start of the project and when bringing in new resources during the project implementation.

The plan must also include systematic evaluation of procedures and regular updating of knowledge about current procedures and tools.

5.3.5	Security Procedures

The Lead Appointed Party must establish a description of security procedures to ensure that the information is handled in a secure manner throughout the project's life cycle. Proactive measures must be taken to prevent unauthorized access to information.

6	Project’s information protocol

Legal aspects related to information are regulated in contract standards for consulting and construction. However, the parties should ensure that the following topics are adequately addressed:

- Rights to use information

- Responsibility for information being correct

- Responsibility for information automatically generated by authoring tools

- Liability for use beyond the model's purpose

- Responsibility for storage during and after the project

- Ranking of contract documents (models, drawings, specifications) and relations between them.

- Common Data Environment (CDE).

- Change, revision and version management

7	Generic project stages

This framework uses the project stages from ISO 29481-1, Annex C as a common reference. For adaptation to a national context, use national project stage standard. Such an adaptation may include collapsing or splitting of stages in the reference table. Please note that any such changes may affect requirements in clause 2.4 Requirements for alphanumerical information.

Table 4 – Generic project stages ISO 29481-1 Annex C

| Principal stage name | Stage No. | Stage name | Description |
| --- | --- | --- | --- |
| Inception | 0 | Portfolio requirements | Establish the need for a project to satisfy the client’s business requirement |
| Brief | 1 | Conception of need | Identify potential solutions to the need and plan for feasibility |
|  | 2 | Outline feasibility | Examine the feasibility of options presented in phase 1 and decide which of these should be considered for substantive feasibility |
|  | 3 | Substantive feasibility | Gain financial approval |
| Design | 4 | Outline conceptual design | Identify major design elements based on the options presented |
|  | 5 | Full conceptual design | Conceptual design and all deliverables ready for detailed planning approval |
|  | 6 | Coordinated design (and procurement) | Fix all major design elements to allow the project to proceed. Gain full financial approval for the project |
| Production | 7 | Production Information | Finalize all major deliverables and proceed to construction. |
|  | 8 | Construction | Produce a product that satisfies all client requirements. Handover the building as planned. |
| Maintenance | 9 | Operation and maintenance | Operate and maintain the product effectively and efficiently. |
| Demolition | 10 | Disposal | Decommission, dismantle and dispose of the components of the project and the project itself according to environmental and health/safety rules |

According to recommended rule for user-defined properties. User-defined properties in addition to the IFC standard shall have a user-defined property set. User-defined property sets should have prefix starting with country code followed by a code signifying the author of the property set and property E.g. EU for common European and HARM for openBIM Harmony. The property should be in English and be logically readable without being too long.