PLAN TEMPLATE

VERSION 1.0 18 March 2025

European openBIM Forum

openBIM Harmony

PART 3.1 BIM EXECUTION PLAN

BUILDINGS

Contents

- General

- BIM Execution Plan

- Common Data Environment

- Responsibilities and routines

- Setup Project Information Model

Explanation of color and type

Black text = Part of framework. Can be distributed as is.

Red text = Example. Shall be changed to project information before distribution

Red italic text = Guidance to fill out information. Shall be deleted before distribution

General

A BIM execution plan is a plan that describes how information is to be managed and exchanged in the project. BEP ensures that all parties agree on how the delivery team must deliver on the appointing party's Exchange Information Requirements and how BIM processes must be carried out, which promotes cooperation and efficiency. The lead appointed party’s project manager is responsible for the BEP being drawn up. It is often created by the lead appointed party’s BIM manager. Source: EN ISO 19650-2.

NB! Tables in clause 2 and 3 should be filled in as part of the offer.

BIM Execution Plan

General

The lead appointed party must respond to the request for information as described in Part 02-1 Exchange Information Requirements. The answer must be given in this BIM Execution Plan. If additional documentation is added to the response beyond this document, these must be referred to from here.

The delivery teams approach to meeting information requirements

Describe methods and solutions that meet requirements in an appropriate way. If the method and solution are described in a separate document, a brief description of this is written in the table below and a separate document is attached and referred to with a link here.

Table 1 - BIM Execution Plan - The delivery team's approach to meeting information requirements

You will find this table in the spreadsheet version in Harmony_02-2_EIR-BEP, tab EIR_LOIN

| EIR Ref. | The delivery team's approach to meeting information requirements |
| --- | --- |
| EIR-01 Project overview |  |
| EIR-02 Modelling and model-based interaction |  |
| EIR-03 Interdisciplinary coordinated and controlled information |  |
| EIR-04 Cost Estimation Information |  |
| EIR-05 Production Information |  |
| EIR-06 Completion and handover of Facility Management Information |  |

The delivery team goals for information delivery

Describe how to measure and document that requirements for information delivery have been met.

The purpose of this is that the appointing party and the lead appointed party agree on the level and scope of the delivery. A description of what is perceived as complete delivery can clarify task understanding and provide an opportunity to clarify any misunderstandings before the project starts.

Table 2 - BIM Execution Plan - The delivery team's goals for information delivery

You will find this table in the spreadsheet version in Harmony_02-2_EIR-BEP, tab EIR_LOIN

| EIR Ref. | The delivery team's goals for information delivery |
| --- | --- |
| EIR-01 Project overview |  |
| EIR-02 Modelling and model-based interaction |  |
| EIR-03 Interdisciplinary coordinated and controlled information |  |
| EIR-04 Cost Estimation Information |  |
| EIR-05 Production Information |  |
| EIR-06 Completion and handover of Facility Management Information |  |

Common Data Environment (CDE)

Table 3 - Common Data Environment (CDE)

You will find this table in the spreadsheet version in Harmony_02-2_EIR-BEP, tab EIR-BEP_CDE

| Purpose of solution | Party responsible for solution | Software |  | Used on hardware If relevant for performance or data security | Responsible for cost |
| --- | --- | --- | --- | --- | --- |
| More solutions are added as needed |  | Software name | Versions | Specification | Role/function |
| Provided by the Appointing Party |  |  |  |  |  |
| Model Management System | Appointing party |  |  |  |  |
| Document Management System | Appointing party |  |  |  |  |
| Issue Management System | Appointing party |  |  |  |  |
| Model viewer | Appointing party |  |  |  |  |
| Document viewers | Appointing party |  |  |  |  |
| Project dashboard | Appointing party |  |  |  |  |
| Room program | Appointing party |  |  |  |  |
| Model requirements (generates machine-interpretable description of model requirements, IDS format) | Appointing party |  |  |  |  |
| Filled in by Lead Appointed Party |  |  |  |  |  |
| Model development architecture | ARC |  |  |  |  |
| Model development landscape architecture | LAN |  |  |  |  |
| Model development of load-bearing constructions | STR |  |  |  |  |
| Model development plumbing | PLU |  |  |  |  |
| Model development mechanical | MEC |  |  |  |  |
| Model development Electro | ELE |  |  |  |  |
| Model check | Appointing party/ Delivery Team/ main contractor |  |  |  |  |
| Communication of model deviations found in model check. | General contractor/ All named parties. |  |  |  |  |
| Cost estimate Construction | ARC/STR/LAN |  |  |  |  |
| Cost calculation plumbing | PLU |  |  |  |  |
| Cost calculation mechanical | MEC |  |  |  |  |
| Cost calculation electrical | ELE |  |  |  |  |
| Cost estimate contractor construction | Main contractor |  |  |  |  |
| Cost calculation contractor plumbing | Main contractor |  |  |  |  |
| Cost calculation contractor mechanical | Main contractor |  |  |  |  |
| Cost estimate contractor electrical | Main contractor |  |  |  |  |

NB! Tables in clauses 4 and 5 are completed in connection with the start of the project and are maintained on an ongoing basis when information is developed.

Responsibilities and routines

General

The Lead appointed party shall establish and implement routines to ensure adequate information management.

Interdisciplinary coordination and model control

Describe the routine for interdisciplinary coordination and model control

Process status coding

Establish and implement a routine for process status coding. Clarify which codes are to be used and share this with all relevant actors in the project.

Change, revision and version control

Describe the routine for handling changes, revision and version. This includes quality assurance, approval and distribution of information to ensure that information is in accordance with the project's requirements before distribution and that there is no doubt about the current revision. The routine includes:

- Complementary and agreed description of steps in the routine.

- Documentation of change history.

- Specification of approval roles and responsibilities.

- Methods for revision and distribution of updated files and models.

See this framework’s part 2.2 Common Data Environment, clause 3.

Training and skills development

Describe the plan for training project personnel, what is to be delivered (EIR), how it is to be delivered (BEP). Everyone who must meet these requirements must be trained. This applies both at the start of the project and when bringing in new resources during the project implementation.

The plan must also include systematic evaluation of routines and regular updating of knowledge about current routines and tools.

Security routines

The lead appointed party must establish a description of security routines to ensure that the information is handled in a secure manner throughout the project's life cycle. Proactive measures must be taken to prevent unauthorized access to information.

Project Information Model setup

Project information

The appointing party provides the project information. This will be used as part of the spatial breakdown structure. In case the project includes more than one site and/or more than one building, expand the table with additional rows to specify all sites and buildings.

You will find the following tables in the spreadsheet version in “Harmony_2-2_EIR-BEP”, tab “BEP_Setup-PIM” (Setup Project Information Model)

Table 4 – Project information

| Information type | Information Information provided by the Appointing Party |
| --- | --- |
| Project name |  |
| Project number |  |
| Site name |  |
| Site number |  |
| Building name |  |
| Building number |  |

Map and elevation datum

The appointing party specifies which map and elevation datum to be utilized by the projects. All participants shall unless otherwise agreed use the same datum.

Table 5 – Map and elevation datum

| Type | Requirement | Description/example |
| --- | --- | --- |
| Map date | Information provided by the Appointing Party | Enter map datum with EPSG code. Also state the appointed party of the map and date of map issue. |
| Altitude datum | Information provided by the Appointing Party | Enter the current altitude datum. E.g. NN2000 (Norwegian example) |
| If more maps are used, more rows are added to the table. |  |  |

Specification of EPSG code in model

Specify the code that communicates map and height datum in the model.

Table 6 – Specification of EPSG Code in model

| Entity | Attribute | Value | String |
| --- | --- | --- | --- |
| IfcProjectedCRS | Name | EPSG:xxxx | IfcProjectedCRS.Name:E PSG:XXXX |
| Example: EPSG:5950 (specifies ETRS89 / NTM zone 10 + NN2000 height). |  |  |  |

Specification of map’s North in model

Correct angel in map is specified with the entity IfcMapConversion and the properties XAxisAbscissa and XaxisOrdinate.

Table 7 – Specification of map’s North in model

| IFC Entity | IFC Property | String | Value1 |
| --- | --- | --- | --- |
| IfcMapConversion | XAxisAbscissa | IfcMapConversion.XAxis Abscissa:1 | 1 |
| IfcMapConversion | XAxisOrdinate | IfcMapConversion.XAxis Ordinate:0 | 0 |
| 1 Fixed values. If values deviate, the model is rotated which is not allowed. |  |  |  |

Specification of project’s point of origin

Project’s point of origin shall be specified in East, North and height. This is specified using the entity IfcMapConversion and the properties for the three dimensions.

Table 8 – Specification of project’s point of origin

| IFC Entity | IFC Property | String |
| --- | --- | --- |
| IfcMapConversion | Eastings | IfcMapConversion.Eastings |
| IfcMapConversion | Northings | IfcMapConversion.Northings |
| IfcMapConversion | OrthogonalHeight | IfcMapConversion.OrthogonalHei ght |

Coordinates of local point of origin and control objects

All disciplines shall use the same common point of origin.

Table 9 – Coordinates of local point of origin and control objects

| Map base | Description |  |  |
| --- | --- | --- | --- |
| Local point of origin in map | X (E): | Y (N): | Z (H): |
| Rotation relative to north | Model rotation is not permitted. Rotation should only be done in model view. |  |  |
| Angle and direction of rotation of model view. |  |  |  |
| Control object 1 | X (E): | Y (N): | Z (H): |
| Control object 2 | X (E): | Y (N): | Z (H): |

Building storeys

All disciplines shall use the same level settings. Specify the project’s common building storeys here.

Table 10 – Building Storeys

| Floor name | Level height | Reference point | Unit |
| --- | --- | --- | --- |
| Rooftop |  |  | Centimeters |
| Floor 9 |  |  | Centimeters |
| Floor 8 |  |  | Centimeters |
| Floor 7 |  |  | Centimeters |
| Floor 6 |  |  | Centimeters |
| Floor 5 |  |  | Centimeters |
| Floor 4 |  |  | Centimeters |
| Floor 3 |  |  | Centimeters |
| Floor 2 |  |  | Centimeters |
| Entrance floor 1 |  |  | Centimeters |
| Lower floor -1 |  |  | Centimeters |
| Lower floor -2 |  |  | Centimeters |

Active proprietary models

Fill in information about active proprietary models. This is important to communicate the totality of the information. Active models are models describing (a part of) the construction.

Table 11 - Active proprietary models

| Discipline E.g. ARC LAN, STR, PLU, MEC, ELE etc. | Model Briefly describe what the model includes. E.g. building, building part etc. | Name proprietary model The name of the file including extension e.g. rvt, dgn, etc. | Date DD.MM.YYYY |
| --- | --- | --- | --- |

Active IFC models

Fill in information about active proprietary models. This is important to communicate the totality of the information. Active models are models describing (a part of) the construction.

Table 12 - Active IFC models

| Discipline E.g. ARC LAN, STR, PLU, MEC, ELE etc. | Model Briefly describe what the model includes. E.g. building, building part etc. | Name IFC The name of the file including format ifc | Date DD.MM.YYYY |
| --- | --- | --- | --- |